# Comcast Equipment List:

## Standard Equipment –

- (1) Primary Access Modem: Broadband Connectivity

- (2) Meraki MX67C-HW-NA: Firewalls in HA

- (1) Meraki MS120-24-HW: Switch

- (1) Meraki MS120-24P-HW: POE Switch

- (2) Meraki MR20-HW: Indoor APs

- (1) WB-250-IPW: Wattbox Power Management

- (2) Panorama Antenna: Ultra-Wideband MiMo Antenna

## Optional Equipment –

- (1) Cradlepoint AER1650 4G Modem: Temporary primary broadband (if needed)

- (1-2) Meraki MR70: Cloud Managed Outdoor AP

- (1) Meraki MS120-24-HW: Additional 24-Port Non-PoE Switch

- (2) Meraki MS120-48-HW: Additional 48-port Non-PoE Switch

- (1) 5u or 9u Cabinet: Equipment Cabinet for Managed Devices.

# Network Diagram

## Standard Deployment

Standard Network Diagram with 48 Port Switch

## Standard + Optional 24-Port Switch

# Network Layout

| VLAN | VLAN NAME | SUBNET | DHCP? | PURPOSE |
| --- | --- | --- | --- | --- |
| 10 | POS-NCR | 192.168.16.0/24 | Yes | Secure CDE Environment for PCI activities. |
|  | POS-XENIAL | 192.168.1.0/24 | No |  |
| 20 | DMB | 192.168.170.0/24 | Yes | Digital Menu Boards, DTD, etc |
| 30 | FRANCHISE-DMZ | 192.168.3.0/24 | Yes | Store devices requiring Internet access |
| 40 | CORP-WIFI | 192.168.4.0/24 | Yes | Store wifi for employees and managers |
| 50 | GUEST-WIFI | 192.168.5.0/24 | Yes | Wifi for customers |
| 70 | KIOSK | 192.168.7.0/24 | Yes | Lobby Ordering Kiosks |
| 90 | DVR | 192.168.9.0/24 | Yes | Dedicated for DVR/Camera systems |
| 99 | FAILOVER | 192.168.99.0/24 | Yes | Dashboard connectivity for FW2 utilizing the broadband connection of FW1. |
| 254 | MANAGEMENT | 192.168.254.0/24 | Yes | Comcast device management for Meraki gear and Wattbox. |

Managed Device Naming Conventions

| Device: | Correct Convention: |
| --- | --- |
| Meraki MX67C (Firewall) | R#PLK00000-FW1 R#PLK00000-FW2 |
| Meraki MS120-24P (POE Switch) | R#PLK00000-SW1 |
| Meraki MS120-24 (Switch) | R#PLK00000-SW2 |
| Meraki MR20 AP (Indoor Access Point) | R#PLK00000-AP1 R#PLK00000-AP2 |
| Meraki MR70 AP (Outdoor Access Point) | R#PLK00000-AP3 |
| WattBox | R#PLK00000-WB1 |

# WIFI/SSIDs List

| SSID | VLAN | Authentication | Pre-Shared Key | Shaping |
| --- | --- | --- | --- | --- |
| Popeyes Internal | 40 | WPA2 w/ PSK | RedBlue2020! | N/A |
| Popeyes Guest | 50 | Splash Page (30m timeout) | N/A | 20Mbps per SSID |
| PLK_SECURE_LB | 10 | Hidden with WPA2 PSK | k2usMX6jup$ne293 | Priority |

Important: PLK_SECURE_LB WIFI network is hidden.  DHCP is not enabled, and static IP addressing is required for any device to connect.

# SwitchPort Mapping

*DMB switches should be plugged into port 11 on each switch.

| R#PLK00000-SW1 |  |  |  | R#PLK00000-SW2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Port | Description | VLAN |  | Port | Description | VLAN |
| Port 1 | POS-BACKOFFICE (NCR) or POS-TERM0 (Xenial) | 10 |  | Port 1 | POS | 10 |
| Port 2 | POS | 10 |  | Port 2 | POS | 10 |
| Port 3 | POS | 10 |  | Port 3 | POS | 10 |
| Port 4 | POS | 10 |  | Port 4 | POS | 10 |
| Port 5 | POS | 10 |  | Port 5 | POS | 10 |
| Port 6 | POS | 10 |  | Port 6 | POS | 10 |
| Port 7 | POS | 10 |  | Port 7 | POS | 10 |
| Port 8 | POS | 10 |  | Port 8 | POS | 10 |
| Port 9 | POS | 10 |  | Port 9 | POS | 10 |
| Port 10 | POS | 10 |  | Port 10 | POS | 10 |
| Port 11 | IDMB-SWITCH | 20 |  | Port 11 | ODMB-SWITCH | 20 |
| Port 12 | DMB | 20 |  | Port 12 | DMB | 20 |
| Port 13 | DMB | 20 |  | Port 13 | DMB | 20 |
| Port 14 | DMB | 20 |  | Port 14 | DMB | 20 |
| Port 15 | DMB | 20 |  | Port 15 | DMB | 20 |
| Port 16 | DMB | 20 |  | Port 16 | DMB | 20 |
| Port 17 | DMZ-MGR-PC | 30 |  | Port 17 | FRANCHISE-DMZ | 30 |
| Port 18 | FRANCHISE-DMZ | 30 |  | Port 18 | FRANCHISE-DMZ | 30 |
| Port 19 | AP-OUTDOOR | TRUNK |  | Port 19 | DVR | 90 |
| Port 20 | AP-1 | TRUNK |  | Port 20 | MX FAILOVER | 99 |
| Port 21 | AP-2 | TRUNK |  | Port 21 | MX FAILOVER | 99 |
| Port 22 | MX-1 UPLINK | TRUNK |  | Port 22 | MX-1 UPLINK | TRUNK |
| Port 23 | MX-2 UPLINK | TRUNK |  | Port 23 | MX-2 UPLINK | TRUNK |
| Port 24 | SW-2 UPLINK | TRUNK |  | Port 24 | SW-2 UPLINK | TRUNK |

48 Port Switch Layout

| SW-1 |  |  |  | SW-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Port | Description | VLAN |  | Port | Description | VLAN |
| Port 1 | POS | 10 |  | Port 1 | POS | 10 |
| Port 2 | POS | 10 |  | Port 2 | POS | 10 |
| Port 3 | POS | 10 |  | Port 3 | POS | 10 |
| Port 4 | POS | 10 |  | Port 4 | POS | 10 |
| Port 5 | POS | 10 |  | Port 5 | POS | 10 |
| Port 6 | POS | 10 |  | Port 6 | POS | 10 |
| Port 7 | POS | 10 |  | Port 7 | POS | 10 |
| Port 8 | POS | 10 |  | Port 8 | POS | 10 |
| Port 9 | POS | 10 |  | Port 9 | POS | 10 |
| Port 10 | POS | 10 |  | Port 10 | POS | 10 |
| Port 11 | POS | 10 |  | Port 11 | POS | 10 |
| Port 12 | POS | 10 |  | Port 12 | POS | 10 |
| Port 13 | POS | 10 |  | Port 13 | POS | 10 |
| Port 14 | POS | 10 |  | Port 14 | POS | 10 |
| Port 15 | POS | 10 |  | Port 15 | POS | 10 |
| Port 16 | POS | 10 |  | Port 16 | POS | 10 |
| Port 17 | POS | 10 |  | Port 17 | POS | 10 |
| Port 18 | POS | 10 |  | Port 18 | POS | 10 |
| Port 19 | POS | 10 |  | Port 19 | POS | 10 |
| Port 20 | POS | 10 |  | Port 20 | POS | 10 |
| Port 21 | POS | 10 |  | Port 21 | DMB | 20 |
| Port 22 | POS | 10 |  | Port 22 | DMB | 20 |
| Port 23 | POS | 10 |  | Port 23 | DMB | 20 |
| Port 24 | IDMB-SWITCH | 20 |  | Port 24 | ODMB-SWITCH | 20 |
| Port 26 | DMB | 20 |  | Port 26 | DMB | 20 |
| Port 27 | DMB | 20 |  | Port 27 | DMB | 20 |
| Port 28 | DMB | 20 |  | Port 28 | DMB | 20 |
| Port 29 | DMB | 20 |  | Port 29 | DMB | 20 |
| Port 30 | DMB | 20 |  | Port 30 | DMB | 20 |
| Port 31 | DMB | 20 |  | Port 31 | DMB | 20 |
| Port 32 | DMB | 20 |  | Port 32 | DMB | 20 |
| Port 33 | DMB | 20 |  | Port 33 | DMB | 20 |
| Port 34 | DMZ-MGR-PC | 30 |  | Port 34 | DMZ-MGR-PC | 30 |
| Port 35 | DMZ-MGR-PC | 30 |  | Port 35 | DMZ-MGR-PC | 30 |
| Port 36 | DMZ-MGR-PC | 30 |  | Port 36 | DMZ-MGR-PC | 30 |
| Port 37 | DMZ-MGR-PC | 30 |  | Port 37 | DMZ-MGR-PC | 30 |
| Port 38 | DMZ-MGR-PC | 30 |  | Port 38 | DMZ-MGR-PC | 30 |
| Port 39 | DMZ-MGR-PC | 30 |  | Port 39 | DMZ-MGR-PC | 30 |
| Port 40 | DVR | 90 |  | Port 40 | DVR | 90 |
| Port 41 | MXFAILOVER | 99 |  | Port 41 | DMZ-MGR-PC | 30 |
| Port 42 | MXFAILOVER | 99 |  | Port 42 | DMZ-MGR-PC | 30 |
| Port 43 | AP-OUTDOOR | TRUNK |  | Port 43 | DMZ-MGR-PC | 30 |
| Port 44 | AP-1 | TRUNK |  | Port 44 | MX FAILOVER | 99 |
| Port 45 | AP-2 | TRUNK |  | Port 45 | MX FAILOVER | 99 |
| Port 46 | MX-1 UPLINK | TRUNK |  | Port 46 | MX-1 UPLINK | TRUNK |
| Port 47 | MX-2 UPLINK | TRUNK |  | Port 47 | MX-2 UPLINK | TRUNK |
| Port 48 | SW-2 UPLINK | TRUNK |  | Port 48 | SW-2 UPLINK | TRUNK |

# Switch Port Mapping For optional 3rd Switch

Important:  If a site has Kiosk, they will be using the switch profile on the right labeled Kiosk

Optional 24 Port Switch                                                          Kiosk 24 Port Switch

| R#PLK00000-SW3 |  |  |  | R#PLK00000-SW3 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Port | Description | VLAN |  | Port | Description | VLAN |
| Port 1 | POS | 10 |  | Port 1 | POS | 10 |
| Port 2 | POS | 10 |  | Port 2 | POS | 10 |
| Port 3 | POS | 10 |  | Port 3 | POS | 10 |
| Port 4 | POS | 10 |  | Port 4 | POS | 10 |
| Port 5 | POS | 10 |  | Port 5 | POS | 10 |
| Port 6 | POS | 10 |  | Port 6 | POS | 10 |
| Port 7 | POS | 10 |  | Port 7 | POS | 10 |
| Port 8 | POS | 10 |  | Port 8 | POS | 10 |
| Port 9 | POS | 10 |  | Port 9 | ORB-Kiosk | 70 |
| Port 10 | POS | 10 |  | Port 10 | Kiosk | 70 |
| Port 11 | POS | 10 |  | Port 11 | Kiosk | 70 |
| Port 12 | POS | 10 |  | Port 12 | Kiosk | 70 |
| Port 13 | DMB | 20 |  | Port 13 | Kiosk | 70 |
| Port 14 | DMB | 20 |  | Port 14 | Kiosk | 70 |
| Port 15 | DMB | 20 |  | Port 15 | Kiosk | 70 |
| Port 16 | DMB | 20 |  | Port 16 | Kiosk | 70 |
| Port 17 | DMB | 20 |  | Port 17 | Kiosk | 70 |
| Port 18 | DMB | 20 |  | Port 18 | DMB | 20 |
| Port 19 | DMB | 20 |  | Port 19 | DMB | 20 |
| Port 20 | FRANCHISE-DMZ | 30 |  | Port 20 | FRANCHISE-DMZ | 30 |
| Port 21 | FRANCHISE-DMZ | 30 |  | Port 21 | FRANCHISE-DMZ | 30 |
| Port 22 | FRANCHISE-DMZ | 30 |  | Port 22 | FRANCHISE-DMZ | 30 |
| Port 23 | SW1-Port 24 UPLINK | TRUNK |  | Port 23 | SW1-Port 24 UPLINK | TRUNK |
| Port 24 | SW2-Port 24 UPLINK | TRUNK |  | Port 24 | SW2-Port 24 UPLINK | TRUNK |

# Device Connectivity

| DEVICE | VENDOR | VLAN | SW1 | SW2 |
| --- | --- | --- | --- | --- |
| POS Back-Office | NCR, rPOS | POS: VL10 | Port 1 | N/A |
| POS Back-Office Printer | NCR, rPOS | POS: VL10 | Ports 1-10 | Ports 1-10 |
| POS Terminal 0 | Xenial | POS: VL10 | Ports 1-10 | Ports 1-10 |
| POS Terminals | NCR, Xenial | POS: VL10 | Ports 1-10 | Ports 1-10 |
| POS Pinpads | Verifone | POS: VL10 | Ports 1-10 | Ports 1-10 |
| POS/Expo Printers | Xenial, (NCR Printers have serial connections) | POS: VL10 | Ports 1-10 | Ports 1-10 |
| Order Ready Screen | Xenial, Stratacache | POS: VL10 | Ports 1-10 | Ports 1-10 |
| KDS/Expo Screens | Xenial | POS: VL10 | Ports 1-10 | Ports 1-10 |
| Drive Thru Director (DTD) | Xenial | DMB: VL20 | Ports 11-16 | Ports 11-16 |
| HME Drive Thru Timer | HME | DMB: VL20 | Ports 11-16 | Ports 11-16 |
| IDMB Players | Xenial, Stratacache | DMB: VL20 | Ports 11-16 | Ports 11-16 |
| IDMB Screens | LG, Samsung | DMB: VL20 | Ports 11-16 | Ports 11-16 |
| ODMB Players | Xenial, Stratacache | DMB: VL20 | Ports 11-16 | Ports 11-16 |
| ODMB Screens | LG, Samsung | DMB: VL20 | Ports 11-16 | Ports 11-16 |
| Order Confirmation Unit (OCU) | Xenial, Stratacache | DMB: VL20 | Ports 11-16 | Ports 11-16 |
| Chicken Chatta | SageNet | FRANCHISE-DMZ: VL30 | Ports 17-18 | Ports 17-18 |
| Managers PC | Various | FRANCHISE-DMZ: VL30 | Ports 17-18 | Ports 17-18 |
| Smart Safe | Various | FRANCHISE-DMZ: VL30 | Ports 17-18 | Ports 17-18 |
| Music/Media Players | Various | FRANCHISE-DMZ: VL30 | Ports 17-18 | Ports 17-18 |
| Security Systems | Various | FRANCHISE-DMZ: VL30 | Ports 17-18 | Ports 17-18 |
| Phone - VOIP/ATA | Various | FRANCHISE-DMZ: VL30 | Ports 17-18 | Ports 17-18 |
| DVR | Various | DVR: VL90 | N/A | Port 19 |

# DVR Considerations

- DVR will be moved to separate network and will need to be reconfigured with the below IP address information for functionality.

  - IP Address: 192.168.9.10

  - Subnet Mask: 255.255.255.0

  - Default-Gateway: 192.168.9.1

  - Primary DNS: 8.8.8.8

  - Secondary DNS: 8.8.4.4